Structural Health Monitoring

of the Hohenzollernbrücke

Using Computational Intelligence Methods

Phase 0: Project Overview, Theory & Dataset Documentation

Course: Computational Intelligence in Engineering (CIE)

Institute of General Mechanics (IAM), RWTH Aachen University

Prof. Bernd Markert, Bhagyashri Bachhav

WS 2023–24 | Revisited 2026

# Table of Contents

(Update this table of contents in Word by right-clicking and selecting “Update Field” after opening the document.)

1. Introduction & Motivation	3

2. Problem Statement	4

3. The Dataset	5

4. Dataset Structure & Labels	6

5. Data Format & Sample Analysis	8

6. Approach & Methodology	10

7. Project Phases	12

8. Environment Setup	14

9. Project Folder Structure	15

# 1. Introduction & Motivation

## 1.1 What is Structural Health Monitoring?

Structural Health Monitoring (SHM) is the process of using sensors, data acquisition systems, and intelligent algorithms to continuously assess the condition of engineering structures such as bridges, buildings, dams, and pipelines. The goal is to detect damage or degradation early, before it leads to catastrophic failure, and to enable data-driven maintenance decisions rather than relying on costly periodic manual inspections.

Traditional inspection methods require engineers to physically examine every part of a structure at scheduled intervals, which is expensive, time-consuming, and often misses internal damage that isn’t visible from the surface. SHM changes this paradigm by providing real-time or near-real-time data about a structure’s behaviour, enabling condition-based maintenance where resources are focused on areas that actually need attention.

## 1.2 Why the Hohenzollernbrücke?

The Hohenzollernbrücke in Cologne (Köln) is one of the busiest railway bridges in Europe. It is a triple-arch steel truss bridge that carries approximately 1,200 trains per day across the Rhine River. Given the enormous volume of traffic and the critical importance of this infrastructure, any structural failure would have severe consequences for transportation, safety, and the economy.

This makes it an ideal candidate for SHM research. The bridge’s complex truss geometry creates interesting structural behaviour under dynamic loading, where damage in one location can manifest as subtle changes in deformation and stress patterns far from the damage site. Detecting and localising these changes from sensor data is exactly the kind of problem that computational intelligence methods are well-suited to solve.

## 1.3 The Role of Computational Intelligence

Computational Intelligence (CI) encompasses machine learning, neural networks, fuzzy systems, and evolutionary computation. In the context of SHM, CI methods are used to learn the relationship between a structure’s measured response (deformations, stresses, accelerations) and its health state (undamaged, damaged at location X, etc.). The key advantage over traditional threshold-based methods is that CI can detect subtle, complex patterns in high-dimensional data that would be impossible for a human to identify manually.

# 2. Problem Statement

## 2.1 The Simulation Setup

An equivalent 3D finite element model of the Hohenzollernbrücke has been created in ANSYS 2023 R2 (Student Edition). The model is a simplified but representative version of the real bridge, capturing the essential truss geometry and structural behaviour. The following loads are applied:

Gravity and self-weight: Standard Earth gravity (9806.6 mm/s²) acting on all structural members.

Train loads: Point forces applied at specific locations on the tracks, simulating trains of varying weight (2000N and 5000N) at different positions (extreme outer track, middle track).

Pedestrian loads: Average distributed loads on the pedestrian walkways.

Crosswind loads: Lateral forces simulating wind acting on the bridge.

The simulation is a transient structural analysis, meaning it captures the dynamic response of the bridge over time (0 to 3 seconds) as loads are applied. The output includes deformations and stresses at every node in the mesh at 11 discrete time steps.

## 2.2 The Task

Given the simulation data, the task has two parts:

Part 1 – Damage Detection (Classification): Determine whether a given set of structural response data comes from a perfect (undamaged) structure or an imperfect (damaged) structure. If damaged, classify which of the 5 imperfection locations the damage belongs to.

Part 2 – Damage Localisation: Identify the exact nodal location of the imperfection within the structure. This goes beyond just classifying the type of damage — it means pinpointing where in the mesh the structural member is missing or weakened.

A test set with unknown condition is provided. The final deliverable is to predict whether this test case represents a perfect or imperfect structure, and if imperfect, identify the location and exact nodes affected.

# 3. The Dataset

## 3.1 Overview

The dataset consists of transient FEA simulation results for the Hohenzollernbrücke under various loading conditions. The data is organised into two main categories:

Perfect structure data: 4 simulation cases representing an undamaged bridge under different loading conditions.

Imperfect structure data: 15 simulation cases representing the bridge with structural imperfections (missing truss members) at 5 different locations, each tested under 3 different loading conditions.

Test set: 1 case with unknown condition, to be predicted by the trained model.

## 3.2 What Each Case Contains

Each simulation case (folder) contains 7 CSV files, each capturing a different physical quantity across all nodes over time:

| File | Quantity | Physical Meaning |
| --- | --- | --- |
| Deformation_X_axis.csv | Ux (mm) | Displacement of each node in the X-direction |
| Deformation_Y_axis.csv | Uy (mm) | Displacement of each node in the Y-direction |
| Deformation_Z_axis.csv | Uz (mm) | Displacement of each node in the Z-direction |
| Total_Deformation.csv | Utotal (mm) | Magnitude of displacement vector: √(Ux²+Uy²+Uz²) |
| Shear_Stress_XY.csv | τxy (MPa) | Shear stress in the XY-plane |
| Shear_Stress_XZ.csv | τxz (MPa) | Shear stress in the XZ-plane |
| Shear_Stress_YZ.csv | τyz (MPa) | Shear stress in the YZ-plane |

Engineering context: Deformations tell us how much the structure displaces under load, while shear stresses tell us about the internal forces within structural members. Together, they provide a comprehensive picture of the structure’s behaviour. Damage (missing members) changes the load paths, which manifests as changes in both deformation patterns and stress distributions.

# 4. Dataset Structure & Labels

## 4.1 Folder Naming Convention

Each folder name encodes the full simulation configuration. The naming convention follows this pattern:

<Condition>_<Imperfection>_<Train Config>_<Track Position>_<Load>

Example: Imperfect_no_3_1_train_extreme_track_5000N

This encodes: Imperfect structure, imperfection at location #3, 1 train on the extreme outer track, 5000N load.

## 4.2 Complete Case Inventory

The following table lists all 20 simulation cases with their extracted labels:

### Imperfect Structure Cases (15 total)

| Imperf. # | # Trains | Track | Load (N) | Label | Node Count |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Extreme | 2000 | imperfect_1 | ~28,900 |
| 1 | 1 | Extreme | 5000 | imperfect_1 | ~28,900 |
| 1 | 2 | Opposite dir. | — | imperfect_1 | ~28,900 |
| 2 | 1 | Extreme | 2000 | imperfect_2 | varies |
| 2 | 1 | Extreme | 5000 | imperfect_2 | varies |
| 2 | 2 | Opposite dir. | — | imperfect_2 | varies |
| 3 | 1 | Extreme | 2000 | imperfect_3 | varies |
| 3 | 1 | Extreme | 5000 | imperfect_3 | varies |
| 3 | 2 | Opposite dir. | — | imperfect_3 | varies |
| 4 | 1 | Extreme | 2000 | imperfect_4 | varies |
| 4 | 1 | Extreme | 5000 | imperfect_4 | varies |
| 4 | 2 | Opposite dir. | — | imperfect_4 | varies |
| 5 | 1 | Extreme | 2000 | imperfect_5 | varies |
| 5 | 1 | Extreme | 5000 | imperfect_5 | varies |
| 5 | 2 | Opposite dir. | — | imperfect_5 | varies |

### Perfect Structure Cases (4 total)

| # Trains | Track | Load (N) | Label | Node Count |
| --- | --- | --- | --- | --- |
| 1 | Extreme | 5000 | perfect | ~29,226 |
| 1 | Extreme | 2000 | perfect | ~29,226 |
| 1 | Middle | 2000 | perfect | ~29,226 |
| 2 | Opposite dir. | — | perfect | ~29,226 |

### Test Set (1 total)

One test case is provided with the same 7-file structure. Its condition (perfect or imperfect) and imperfection location (if any) are unknown and must be predicted.

## 4.3 Key Observation: Node Count Variation

An important detail: the perfect structure has approximately 29,226 nodes, while imperfect structures have fewer (approximately 28,901 for imperfection #1). This is because the imperfection involves removing a structural member, which removes the associated nodes from the mesh. This node count difference is itself a potential feature — but more importantly, it means we cannot simply do a direct node-to-node comparison without first aligning the node sets.

# 5. Data Format & Sample Analysis

## 5.1 CSV File Format

Each CSV file follows an identical structure. Taking Deformation_X_axis.csv as an example:

| Property | Value |
| --- | --- |
| Rows | ~28,901 (one per node) |
| Columns | 12 (1 node number + 11 time steps) |
| Time steps | 0.0, 0.1, 0.5, 0.9, 1.1, 1.5, 1.9, 2.1, 2.5, 2.9, 3.0 seconds |
| Units | Deformation in mm, Stress in MPa |
| Data format | Scientific notation (e.g., -2.7458e-005) |
| File encoding | CSV with trailing comma on each row |

## 5.2 Time Step Interpretation

The 11 time steps are not uniformly spaced. Looking at the progression (0, 0.1, 0.5, 0.9, 1.1, 1.5, 1.9, 2.1, 2.5, 2.9, 3.0), the simulation captures the transient response at moments designed to capture key loading phases. The pattern suggests three loading phases of roughly 1 second each (0–0.9s, 1.1–1.9s, 2.1–2.9s), with boundary snapshots at t=0 and t=3.0.

Practical significance: The values at t=0.0 and t=3.0 are nearly identical and very small (order of 10⁻⁵ mm), representing the initial and final rest states. The largest deformations occur during the loading phases, with the peak typically at t=2.5s.

## 5.3 Sample Statistical Analysis

Analysis of the Deformation_X_axis.csv file from the case Imperfect_no_1_1_train_extreme_track_5000N reveals the following statistics:

| Time (s) | Mean | Std Dev | Min | Max | ¦Max¦ | RMS |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0 | -2.8e-5 | 2.5e-5 | -8.2e-5 | 3.6e-5 | 8.2e-5 | 3.7e-5 |
| 0.1 | -0.0167 | 0.0513 | -0.1369 | 0.0355 | 0.1369 | 0.0540 |
| 0.5 | -0.0333 | 0.1025 | -0.2711 | 0.0706 | 0.2711 | 0.1078 |
| 0.9 | -0.0167 | 0.0513 | -0.1369 | 0.0355 | 0.1369 | 0.0540 |
| 1.5 | -0.0267 | 0.0474 | -0.1439 | 0.0340 | 0.1439 | 0.0544 |
| 2.5 | -0.0521 | 0.1337 | -0.3740 | 0.0725 | 0.3740 | 0.1434 |
| 3.0 | -2.8e-5 | 2.5e-5 | -8.2e-5 | 3.6e-5 | 8.2e-5 | 3.7e-5 |

## 5.4 Key Observations from Initial Analysis

1. Scale of values: Deformations are sub-millimetre, ranging from approximately 10⁻⁷ mm (at rest) to 0.374 mm (peak). This is entirely expected for a large steel bridge under operational loads. The small magnitudes mean numerical precision and appropriate normalisation will be critical during preprocessing.

2. Peak response at t=2.5s: The maximum absolute deformation consistently occurs at t=2.5s across the nodes. This suggests that the third loading phase (2.1–2.9s) applies the heaviest loading.

3. Symmetry at rest states: The values at t=0.0 and t=3.0 are nearly identical, confirming that the structure returns to its initial (gravity-only) state after the transient loads pass. The values at t=0.1 and t=0.9 are also very similar, as are t=2.1 and t=2.9, suggesting symmetric loading within each phase.

4. Value range spans 6 orders of magnitude: From 10⁻⁷ to 10⁻¹ mm. This wide dynamic range suggests that logarithmic scaling or robust normalisation (e.g., StandardScaler after clipping) may be beneficial during preprocessing.

5. Node count variation: Perfect structure: ~29,226 nodes. Imperfect case #1: 28,901 nodes. The difference (~325 nodes) corresponds to the removed structural member. This information is itself diagnostic.

# 6. Approach & Methodology

## 6.1 The Core Challenge

This problem has a fundamental challenge: we have very few samples (19 training cases + 1 test case) but each sample is extremely high-dimensional (7 files × ~29,000 nodes × 11 time steps = ~2.25 million values per case). This is a classic “small n, large p” problem in machine learning, where the number of features vastly exceeds the number of samples.

We cannot feed all 2.25 million raw values directly into a model — it would catastrophically overfit on 19 samples. Instead, we need a carefully designed feature engineering pipeline that compresses the raw data into a compact but informative representation. The quality of these features will determine the success of the entire project.

## 6.2 Strategy: Comparison Against Baseline

The most effective strategy for damage detection in SHM is to compare the response of a potentially damaged structure against a known healthy (perfect) baseline. For each loading condition, we have both a perfect and an imperfect simulation. The key idea is:

Difference = Imperfect Response − Perfect Response (under same loading)

If the structure is perfect, this difference should be zero (or very close to zero). If it is damaged, the difference will be non-zero and will be concentrated near the damage location. This is the foundation of our approach.

## 6.3 Feature Engineering Strategy

From each simulation case, we will extract features at multiple levels:

Global features (per case): Summary statistics (max, mean, std, RMS) of deformations and stresses across all nodes. These capture the overall structural response and can distinguish between different loading conditions and damage severities.

Temporal features (per case): How the response evolves over the 11 time steps. Peak timing, rate of change, and energy metrics (sum of squared values over time) can reveal dynamic behaviour changes caused by damage.

Spatial features (per node): Node-level differences between the imperfect and perfect cases. For damage localisation, these are critical — the nodes near the damage will show the largest differences.

Cross-quantity features: Ratios and relationships between different quantities (e.g., stress-to-deformation ratios) that may be more sensitive to damage than any single quantity alone.

## 6.4 Modelling Strategy

### For Damage Detection (Classification)

Baseline model — Random Forest / XGBoost: These ensemble methods work well with small tabular datasets, are robust to overfitting when properly tuned, and provide feature importance rankings that are invaluable for understanding what drives the classification. This will be our first model.

Neural network model — Feedforward NN or 1D-CNN: As the course focuses on neural architectures, we will also build an ANN. A feedforward network on extracted features is the pragmatic choice; a 1D-CNN over the time-series data (11 time steps) is a more advanced option that can learn temporal patterns directly.

Dealing with small sample size: With only 19 training samples, we will use leave-one-out cross-validation (LOOCV) to maximise training data at each fold, and will be aggressive about regularisation (dropout, early stopping, L2 penalty) to prevent overfitting.

### For Damage Localisation

Difference-based approach: Compute the node-level response difference between the test case and the matching perfect case. Nodes with the largest absolute difference are the likely damage locations. This is simple but effective and directly interpretable.

Anomaly detection: Alternatively, train a model on the node-level statistics of perfect cases to establish “normal” behaviour, then flag nodes in the test case that deviate significantly. Isolation Forest or autoencoders are natural choices here.

## 6.5 Evaluation Metrics

Following the project requirements (slide 11 of the course presentation), we will compute:

For classification: Confusion matrix, ROC curves, AUC scores, precision, recall, and F1-score. Given the multi-class nature (6 classes: perfect + 5 imperfection locations), we will use macro-averaged metrics.

For localisation: Visual comparison of predicted damage location against known imperfection positions. We can quantify this as the distance (in node indices or spatial coordinates) between predicted and actual damage locations.

Model comparison: We will compare multiple models using the same cross-validation scheme and present results in a unified table.

# 7. Project Phases

The project is structured into distinct phases, each building on the previous one. This section provides a roadmap of what each phase covers and why.

Phase 0 — Project Setup & Documentation (this document) Objective: Understand the problem, document the dataset, set up the development environment, and establish the project structure. Deliverables: This documentation, conda environment, project folder structure. Status: Current phase

Phase 1 — Data Loading, Exploration & Visualisation Objective: Load all 20 cases programmatically, verify data integrity, compute per-case statistics, and visualise the structural response patterns. Key questions to answer: How does the response differ between perfect and imperfect cases? Are the differences visible in summary statistics? What does the spatial distribution of deformation look like? Deliverables: Data loading pipeline, exploratory analysis notebook, initial visualisations.

Phase 2 — Feature Engineering & Preprocessing Objective: Transform raw simulation data into a compact, informative feature matrix suitable for machine learning. This is the most critical phase for model performance. Techniques: Statistical aggregation, baseline differencing, normalisation, dimensionality reduction (PCA), feature selection. Deliverables: Feature extraction pipeline, feature matrix (samples × features), feature analysis.

Phase 3 — Classification (Damage Detection) Objective: Build and evaluate classifiers to detect damage and identify the imperfection location from structural response data. Models: Random Forest (baseline), XGBoost, Feedforward Neural Network, 1D-CNN (advanced). Cross-validation with LOOCV due to small sample size. Deliverables: Trained models, confusion matrices, ROC curves, model comparison table.

Phase 4 — Damage Localisation Objective: Pinpoint the exact location (nodes) of the structural imperfection within the bridge mesh. Methods: Baseline differencing at node level, anomaly detection (Isolation Forest, autoencoder), spatial visualisation of damage indicators. Deliverables: Damage localisation maps, predicted damage locations for test set.

Phase 5 — Test Set Prediction & Final Report Objective: Apply the best-performing model to the test set and present a final prediction with confidence metrics. Deliverables: Test set prediction, final documentation, GitHub-ready repository.

# 8. Environment Setup

## 8.1 Conda Environment

We use Anaconda to create an isolated Python environment for this project. This ensures reproducibility and avoids dependency conflicts. Here are the setup commands:

conda create -n shm python=3.11 -y

conda activate shm

pip install numpy pandas matplotlib seaborn scikit-learn xgboost

pip install torch torchvision  # for neural network models

pip install jupyter notebook  # for interactive exploration

## 8.2 Why These Libraries?

numpy & pandas: Core data manipulation. Pandas DataFrames are ideal for the tabular CSV data; NumPy for fast numerical operations on the large arrays.

matplotlib & seaborn: Visualisation. Matplotlib for fine-grained control over plots; Seaborn for statistical visualisations like heatmaps and distribution plots.

scikit-learn: The workhorse ML library. Provides Random Forest, cross-validation utilities, preprocessing (StandardScaler, PCA), and evaluation metrics (confusion matrix, ROC, AUC).

xgboost: Gradient boosting implementation. Often outperforms Random Forest on structured/tabular data and provides excellent feature importance analysis.

PyTorch: Deep learning framework. More flexible and Pythonic than TensorFlow, preferred for research and custom architectures. We’ll use this for the neural network models.

## 8.3 IDE Setup

VS Code is an excellent choice for this project. For the exploratory and iterative nature of the work, I recommend using Jupyter notebooks (.ipynb files) within VS Code for Phases 1–2 (exploration and feature engineering), as they allow you to run code cell-by-cell, see outputs inline, and iterate quickly. For Phases 3–5 (modelling and production code), structured Python scripts (.py files) are better as they’re cleaner, version-controllable, and easier to refactor.

VS Code has excellent Jupyter support via the “Jupyter” extension. Install it from the Extensions marketplace, then select the “shm” conda environment as the kernel.

# 9. Project Folder Structure

The following structure is recommended for the GitHub repository:

structural-health-monitoring/

│

├── data/

│   ├── perfect_structure_data/         # 4 perfect cases

│   │   ├── Perfect_structure_1_train_.../  # Each contains 7 CSVs

│   │   └── ...

│   ├── imperfect_structure_data/       # 15 imperfect cases

│   │   ├── Imperfect_no_1_.../            # Each contains 7 CSVs

│   │   └── ...

│   └── Test_set/                       # 1 test case (7 CSVs)

│

├── notebooks/                          # Jupyter notebooks

│   ├── 01_data_exploration.ipynb

│   ├── 02_feature_engineering.ipynb

│   └── 03_visualisation.ipynb

│

├── src/                                # Python modules

│   ├── data_loader.py                  # Functions to load & parse data

│   ├── features.py                     # Feature extraction pipeline

│   ├── models.py                       # Model definitions

│   └── evaluate.py                     # Evaluation & plotting utilities

│

├── docs/                               # Documentation

│   ├── Phase_0_Project_Overview.docx   # This document

│   └── ...                             # Phase docs added as we go

│

├── models/                             # Saved trained models

├── results/                            # Output figures, metrics

├── environment.yml                     # Conda environment export

├── README.md                           # Project overview for GitHub

└── .gitignore                          # Exclude data/, models/, etc.

Note on .gitignore: The data/ folder should be listed in .gitignore since the simulation files are large and potentially proprietary. Include a README inside data/ explaining how to obtain the dataset. Similarly, the models/ folder should be gitignored since trained model files are large and reproducible from the code.

End of Phase 0 Documentation

Next: Phase 1 — Data Loading, Exploration & Visualisation